BBR-SAT: Ephemeris-Driven Cross-Orbit BDP Context-Switching for QUIC Congestion Control

Rajat Bhambani

Independent Research Contribution

DRAFT v5 — GLOBECOM 2026 Workshop — Deadline: 12 August 2026

[NOTE — v5 CHANGELOG: (1) RTT/BW epistemological separation: RTT trusted from ephemeris, BW treated as ceiling with immediate ProbeBW validation. (2) Fairness sub-experiment added per RFC 9743/BCP 133. (3) PPB reframed: 'motivated by' not 'extending.' (4) Ephemeris pathway scoped to operator-administered endpoints. (5) RFC 9743 obsoletes RFC 5033 noted. (6) All v3/v4 fixes preserved.]

Abstract

Abstract — Satellite congestion control research has converged on intra-LEO handover: StarQUIC, SATPIPE, LeoCC, and Creo all target single-constellation dynamics. The IETF CCWG's draft-lai-ccwg-lsncc formalizes the underlying problem as the Path Phase Boundary (PPB) — a reactive, parameter-free advisory that path conditions may have changed discontinuously — but specifies no CCA mechanism. Separately, Careful Resume accelerates connection startup from cached parameters but operates exclusively at connection establishment. None of this work addresses cross-orbit transitions: a mid-connection shift between orbit classes (LEO, MEO, GEO) producing a simultaneous step change in round-trip time and bottleneck bandwidth. BBRv3's 10-second minimum-RTT filter creates a documented blind spot on such transitions. We propose BBR-SAT, a minimal extension to BBRv3 motivated by the PPB problem statement. BBR-SAT maintains a per-orbit-class state table and switches between entries on a handover signal. A key design insight separates what ephemeris can determine — propagation delay, which is geometric and deterministic — from what it cannot — available bandwidth, which depends on real-time beam loading. Accordingly, BBR-SAT trusts the pre-loaded min_rtt immediately but treats the pre-loaded max_bw as a pacing-rate ceiling subject to immediate validation by BBRv3's ProbeBW cycle. We evaluate BBR-SAT in picoquic against vanilla BBRv3, Careful Resume, Creo-style and StarQUIC-style baselines across 189 single-flow conditions and a multi-flow fairness experiment, with full statistical rigor. Results [PLACEHOLDER].

Index Terms — BBR, BBRv3, QUIC, congestion control, satellite, NTN, cross-orbit handover, ephemeris, bandwidth-delay product, context switching.

# I. Introduction

Internet congestion control was not designed for paths that change their fundamental characteristics mid-connection. Satellite non-terrestrial networks (NTN) break this assumption. A connection migrating from a LEO satellite path (~25 ms one-way delay, tens of Mbps) to a GEO path (~290 ms one-way, single-digit Mbps) experiences a simultaneous 10× step in RTT and 5× reduction in bottleneck bandwidth.

The satellite transport community has responded to part of this problem. StarQUIC [1] freezes the congestion window around Starlink's 15-second handover cycle. SATPIPE [2] drains the queue before intra-LEO handovers. LeoCC [3] detects handovers via ICMP probes. OrbCC [4] uses in-network telemetry. The Creo algorithm [5] (Yan et al., ICC 2025) uses DRL-based cross-layer advance notifications to pause and resume TCP during LEO handovers. All operate within a single LEO constellation. None address cross-orbit transitions.

The IETF Congestion Control Working Group (CCWG) has begun formalizing the problem. Lai et al.'s draft-lai-ccwg-lsncc-01 [6] proposes the Path Phase Boundary (PPB) — 'an advisory indication that path conditions may have changed discontinuously such that previously-learned bandwidth and baseline-delay estimates may become stale.' PPB is reactive (detected post-event via host-side probes), parameter-free (carries no RTT/BW), and CCA-agnostic (proposes no algorithm). Its normative guidance is one sentence: endpoints SHOULD avoid combining measurements across boundaries.

Separately, Careful Resume [7] (RFC Editor Queue) caches observed (cwnd, RTT) from a prior connection for fast startup. It stores one state per remote endpoint, operates at connection establishment only, and never re-engages mid-connection. The sole satellite evaluation [8] tests CUBIC, not BBR.

BBRv3 [9] has an inherent asymmetry: bandwidth decreases are detected in sub-seconds via inflight tracking; RTT increases are gated by the min_rtt filter (BBR.MinRTTFilterLen = 10 s). Mazilu et al. [10] independently validate that BBRv3 'reacts slowly to abrupt RTT shifts.' For a LEO→GEO transition (50 ms → 580 ms path RTT), BBRv3 paces against stale min_rtt for up to 10 seconds.

We propose BBR-SAT, a minimal BBRv3 extension motivated by the PPB problem statement [6]. BBR-SAT addresses the gap left by all three lines of prior work: intra-LEO cross-layer mechanisms (freeze/pause but don't context-switch across orbit tiers), PPB (defines the boundary abstraction but specifies no CCA response), and Careful Resume (connection-start only). A critical design insight drives the mechanism: orbital ephemeris determines propagation delay (geometry) but not available bandwidth (load). Therefore BBR-SAT trusts the pre-loaded min_rtt immediately — eliminating the 10-second filter blind spot — but treats the pre-loaded max_bw as a pacing-rate ceiling, relying on BBRv3's own ProbeBW cycle to discover the actual available bandwidth within 1–2 RTTs.

Our contributions:

- A per-orbit-class state table of (min_rtt, max_bw) tuples, with an epistemological separation: RTT is trusted from ephemeris; BW is a ceiling estimate subject to immediate in-band validation.

- A handover signal that resets BBRv3's min_rtt filter and caps the pacing rate, with three concrete delivery pathways: ephemeris-derived prediction (within the constellation operator's domain), in-network telemetry, and host-side probe detection as a reactive fallback.

- A bandwidth-triggered DRAIN that throttles the pacing rate when transitioning to a lower-bandwidth orbit.

- A controlled evaluation against five baselines across 189 single-flow conditions plus a multi-flow fairness experiment per RFC 9743/BCP 133 [11].

# II. Background and Related Work

## A. BBRv3 Filter Mechanics

BBRv3 [9] (draft-ietf-ccwg-bbr-05, March 2026): BBR.MinRTTFilterLen = 10 s. BBR.MaxBwFilterLen = 2 ProbeBW cycles. ProbeRTTInterval = 5 s, ProbeRTTDuration = 200 ms, ProbeRTTCwndGain = 0.5, LossThresh = 2%, Beta = 0.7.

## B. The Path Phase Boundary Problem

Lai et al. [6] (draft-lai-ccwg-lsncc-01, Feb 2026, CCWG, individual, Informational) define the PPB abstraction. The draft is deliberately mechanism-agnostic: §5 states endpoints may infer PPBs or networks may signal them. The only concrete instantiation is a host-side ICMP-probe heuristic. BBR-SAT is motivated by this problem statement: Lai et al. identify the discontinuous-path-change problem and recommend that CCAs respect boundaries; we provide a concrete BBRv3 mechanism that does so. We do not claim to instantiate or extend PPB — our signal carries parameters (target orbit class) and triggers a specific CCA response, both of which PPB's parameter-free, CCA-agnostic design deliberately excludes.

[NOTE — v5 CHANGE: Reframed from 'extending PPB' to 'motivated by.' This is honest — we're responding to the problem lsncc defines, not co-opting its terminology.]

## C. Careful Resume

Careful Resume [7] (-24, RFC Editor Queue): caches one (cwnd, RTT) per remote endpoint. Operates at connection start only. We include CR as a baseline to empirically confirm that IETF transport standards intentionally leave mid-connection satellite mobility unaddressed.

## D. Intra-LEO Cross-Layer Congestion Control (2024–2026)

| Mechanism | Venue | Mid-Conn? | Cross-Orbit? | Signal | CCA Approach |
| --- | --- | --- | --- | --- | --- |
| PPB [6] | IETF CCWG -01 | Reactive | No | ICMP probes | Advisory only |
| Careful Resume [7] | IETF TSVWG -24 | No | No | Cached obs. | Generic |
| StarQUIC [1] | LEO-NET@MobiCom'24 | Yes | No | 15-s schedule | cwnd-freeze |
| SATPIPE [2] | INFOCOM'25 | Yes | No | NTP schedule | Queue drain |
| LeoCC [3] | SIGCOMM'25 | Yes | No | ICMP probe | Reconfig model |
| Creo [5] | ICC 2025 | Yes | No | Cross-layer | DRL+freeze |
| OrbCC [4] | arXiv (unreview.) | Yes | No | INT | BDP target |
| BBR-SAT | This paper | Yes | Yes | Ephemeris/INT/probe | BDP ctx switch |

BBR-SAT's novelty is not the cross-layer signal (Creo [5] already uses one) — it is the algorithmic response: a multi-tier BDP context switch across heterogeneous orbit classes, with an epistemological separation between deterministic RTT (trusted) and dynamic BW (ceiling).

Additional work: Ohmori et al. [12] (AINTEC '25) improve on StarQUIC. Garcia et al. [13] (ANRW '25) compare TCP CCAs over Starlink. OrbitTransit [14] (Zhao et al., Simon Fraser, MobiSys 2026 Emerging Ideas) is a network-layer traffic diffusion system — orthogonal to transport-layer CCA.

## E. IETF Satellite Transport Landscape

RFC 9743/BCP 133 [11] (Duke & Fairhurst, March 2025; obsoletes RFC 5033) is the normative framework for specifying new CCAs; it explicitly names satellite links as problematic. draft-jones-tsvwg-transport-for-satellite-02 [15] (expired, co-authored by Fairhurst/Kuhn — probable reviewers) updates RFC 2488/3135 for QUIC. Three TSVWG drafts (Yang [16,17]; Yue [18]; all expired) form a problem/requirements/use-cases cluster anchoring a proposed T4SAT BOF. IRTF SPACERG [19] (chartered Jan 2025) is the natural research-track venue.

## F. Architectural Assumption

BBR-SAT is architecture-agnostic. It requires that a single QUIC connection can experience a cross-orbit path change. This is satisfied by regenerative-payload satellites with transport proxies (under study in 3GPP NTN), multi-orbit ground gateways, or future B5G/6G space-edge architectures. We decouple from any specific 3GPP release.

# III. BBR-SAT Design

## A. Per-Orbit State Table

A fixed-size table indexed by orbit class (LEO=0, MEO=1, GEO=2). Each entry:

- min_rtt_us: Minimum RTT (μs). Initialized from ephemeris (deterministic). Trusted immediately on swap.

- max_bw_bps: Bottleneck bandwidth (bps). Initialized from provisioned beam capacity. Treated as a pacing-rate ceiling, not a committed rate. Subject to immediate ProbeBW validation.

- bw_confidence: EPHEMERIS (ceiling only) or MEASURED (from learning loop — trusted for 1 RTT before re-probing).

- valid, last_updated: Staleness tracking.

The epistemological separation: Orbital mechanics give you geometry — propagation delay is determined by altitude and slant range, computable to sub-millisecond precision from TLE/SGP4. Orbital mechanics do not give you telemetry — available bandwidth depends on beam loading, cross-traffic, rain fade, and scheduling, all of which vary in real time. Therefore: trust RTT from ephemeris; treat BW from ephemeris as a ceiling. After the min_rtt swap, BBRv3's normal ProbeBW cycle (starting from ProbeBW_REFILL → ProbeBW_UP → ProbeBW_DOWN) discovers the actual available bandwidth within 1–2 RTTs of the new path. If the beam is congested and only 2 Mbps of a provisioned 10 Mbps is available, ProbeBW_DOWN detects the loss/queueing signal and pulls the rate down within one cycle. The pre-loaded max_bw prevents the sender from probing above the provisioned capacity — it bounds the upside, not the downside.

[NOTE — v5 CHANGE: This is the critical design fix from Gemini R3. The prior design trusted both RTT and BW from ephemeris equally. That was wrong — BW is not deterministic from orbital geometry. The new design trusts RTT (geometric, deterministic) and treats BW as a ceiling (provisioned, possibly stale). ProbeBW validates the actual available BW within 1–2 RTTs.]

## B. Handover Signal and Signaling Pathways

BBR-SAT's handover trigger is HANDOVER_PREDICTED(target_orbit, lead_time). This is a concrete, parameterized handover signal motivated by the PPB problem statement [6] but structurally distinct from PPB: it is predictive (pre-event), parameterized (carries target orbit class), and triggers a specific CCA response (BDP context switch).

Signaling pathways, ordered by information richness:

- Ephemeris-derived prediction: The constellation routing plane computes upcoming orbit-class transitions from orbital mechanics and delivers HANDOVER_PREDICTED to the transport endpoint. This pathway requires the transport endpoint to reside within the constellation operator's administrative domain (e.g., on-board proxy, gateway proxy, or operator-hosted edge). It does not apply to arbitrary third-party cloud endpoints.

- In-network telemetry: Per-hop delay annotations in the packet header (à la OrbCC [4]). The transport endpoint infers an imminent transition from sustained per-hop delay shifts. Applicable across administrative boundaries.

- Host-side probe detection (reactive fallback): The ICMP-probe heuristic described in draft-lai-ccwg-lsncc §4.2 [6]. Reactive (lead_time = 0). Available to any endpoint. The range study in §V.B evaluates this degraded mode.

[NOTE — v5 CHANGE: Pathway (a) now explicitly scoped to operator-administered endpoints. This closes the 'how does AWS get the signal' gap Gemini raised.]

On receiving HANDOVER_PREDICTED:

- Save: Write converged (BBR.min_rtt, BBR.max_bw) to current orbit's table entry.

- Load min_rtt: Overwrite BBR.min_rtt with target orbit's value. Reset BBR.min_rtt_stamp. This is trusted — RTT is deterministic from ephemeris.

- Load max_bw as ceiling: Set BBR.max_bw to target orbit's value. This is a pacing-rate cap, not a committed rate. If bw_confidence = EPHEMERIS, immediately enter ProbeBW_REFILL to discover actual available BW. If bw_confidence = MEASURED (from a prior traversal via learning loop), trust for 1 RTT before re-probing.

- BW-triggered DRAIN: If BW_target < BW_current, enter 1-RTT DRAIN. Prevents queue overflow at the narrower bottleneck.

Why bandwidth, not BDP? LEO BDP = 312 KB; GEO BDP = 725 KB. GEO has higher BDP because RTT inflation outweighs BW reduction. A BDP trigger would not fire on LEO→GEO. The correct trigger is BW_old > BW_new.

## C. Learning Loop

After one full ProbeBW cycle on the new orbit, update the table entry with observed (min_rtt, max_bw) and set bw_confidence = MEASURED. Subsequent handovers to this orbit load measured values — trusted for 1 RTT, then re-probed. The table is self-correcting.

## D. Lead-Time Range Study

We evaluate lead times 0–60 s. At lead_time = 0, BBR-SAT operates reactively (pathway 3). The delta between lead_time = 0 and lead_time > 0 quantifies the value of predictive over reactive boundary detection.

[PLACEHOLDER — Figure 1 — BBR-SAT state machine: per-orbit table with bw_confidence, handover signal, BW-triggered DRAIN, learning loop.]

# IV. Evaluation Methodology

## A. Simulation Tool

picoquic [20] with BBRv3, virtual-time simulator. Based on FAU/Aberdeen fork (Careful Resume for CUBIC [8]). Extended with: CR→BBR port, BBR-SAT, Creo-style and StarQUIC-style baselines.

## B. Path Model

| Parameter | LEO | MEO | GEO |
| --- | --- | --- | --- |
| One-way delay (ms) | 25 | 80 | 290 |
| RTT (ms) | 50 | 160 | 580 |
| Bottleneck BW (Mbps) | 50 | 30 | 10 |
| BDP (bytes) | 312,500 | 600,000 | 725,000 |
| Queue (× BDP) | 1.0 | 1.0 | 1.0 |

BDP ordering: LEO lowest (312 KB), GEO highest (725 KB). DRAIN triggers on BW reduction.

## C. Baselines

- B1 — Vanilla BBRv3. Stock picoquic BBR.

- B2 — BBRv3 + Careful Resume. Documents the IETF gap.

- B3 — BBRv3 + cwnd-freeze (StarQUIC-style).

- B4 — BBRv3 + cross-layer pause (Creo-style).

- BBR-SAT. Full mechanism per §III.

## D. Experiment 1: Single-Flow Performance

| Variable | Values | Count |
| --- | --- | --- |
| Orbit transition | LEO→GEO, LEO→MEO, MEO→GEO | 3 |
| Handover timing | T = 30, 60, 120 s | 3 |
| Lead time | 0, 2, 5, 10, 20, 30, 60 s | 7 |
| Non-congestion loss | 0%, 0.5%, 2% (Gilbert-Elliott) | 3 |

Conditions: 3 × 3 × 7 × 3 = 189 × 5 baselines = 945 configs × 30 runs = 28,350 runs.

Dependent variables: T90 (time-to-90%-throughput, primary), cumulative goodput (60 s window), peak queue depth (±5 s), congestion-induced loss (±5 s).

## E. Experiment 2: Multi-Flow Fairness (per RFC 9743)

RFC 9743/BCP 133 [11] requires evaluation of fairness and protection against adverse interactions with existing flows. We test two competing flows sharing a single bottleneck through the handover window:

- Scenario F1: BBR-SAT vs vanilla BBRv3. Both experience the same path change.

- Scenario F2: BBR-SAT vs CUBIC. Both experience the same path change.

- Scenario F3: BBR-SAT (experiencing handover) vs vanilla BBRv3 (steady-state, no handover signal). Tests whether BBR-SAT's BDP swap starves a co-located flow that is still adapting normally.

Fairness variables: 3 scenarios × 3 orbit transitions × 3 loss rates = 27 conditions × 30 runs = 810 runs. Dependent variable: Jain's fairness index J(x) over the 60 s handover window, plus per-flow throughput share.

Total simulation budget: 28,350 (Exp 1) + 810 (Exp 2) = 29,160 runs. ~10.8 hours on a 4-core N100 with 2× re-run buffer.

[NOTE — v5 CHANGE: Experiment 2 is entirely new, added per RFC 9743 fairness requirements. Scenario F3 is the critical one — it tests whether BBR-SAT's pre-loaded BDP harms a naive competing flow.]

## F. Statistical Protocol

30 runs per condition, distinct seeds, 95% CI. Median + IQR for T90, mean ± CI for tables. Jain's index reported as mean ± CI.

# V. Results

## A. LEO→GEO: Headline Result

[PLACEHOLDER — Figure 2 — Sending rate vs time, LEO→GEO, T=30s, lead_time=5s. Five traces.]

[PLACEHOLDER — Table I — T90, goodput, peak queue, loss. Five baselines. Mean ± 95% CI.]

[Narrative TBD.] Critical null result: B2 (CR) = B1 (vanilla) post-handover.

## B. Lead-Time Range Study

[PLACEHOLDER — Figure 3 — T90 vs lead_time (0–60 s). Delta between 0 and 5 quantifies predictive vs reactive value.]

## C. BW Ceiling Validation

[PLACEHOLDER — Figure 4 — Sending rate vs time for BBR-SAT with congested GEO beam (actual 2 Mbps, provisioned 10 Mbps). Show ProbeBW_DOWN pulling rate from 10→2 Mbps within 1–2 RTTs. This validates the ceiling design.]

[NOTE — v5 CHANGE: This subplot is new — directly answers the 'what if the beam is congested' objection. Run with BW_actual = 0.2 × BW_provisioned for one representative condition.]

## D. Non-Congestion Loss

[PLACEHOLDER — Figure 5 — T90 vs loss rate for each baseline.]

## E. MEO Transitions

[PLACEHOLDER — Table II — T90 for LEO→MEO and MEO→GEO.]

## F. Multi-Flow Fairness

[PLACEHOLDER — Table III — Jain's fairness index for F1 (BBR-SAT vs BBRv3), F2 (BBR-SAT vs CUBIC), F3 (BBR-SAT handover vs BBRv3 steady-state). LEO→GEO, 0% loss. Mean ± 95% CI.]

[PLACEHOLDER — Figure 6 — Per-flow throughput share over time for F3 (the critical scenario). Show whether BBR-SAT's pre-loaded BDP temporarily starves the competing vanilla BBRv3 flow.]

[Narrative TBD.] Expected: BBR-SAT's BW ceiling should prevent it from claiming more than its fair share at the new bottleneck. The ceiling caps the upside; ProbeBW discovers the fair share. If J(x) < 0.9 during the transition window, we have a fairness problem that needs algorithmic mitigation (e.g., slower ramp from ceiling to measured).

# VI. Discussion and Limitations

The epistemological separation is the core insight. Ephemeris gives geometry; real-time probing gives telemetry. BBR-SAT trusts the former and validates the latter. This is not merely a defensive concession — it is a design principle that makes BBR-SAT safe to deploy in congested multi-tenant satellite networks. The pre-loaded min_rtt eliminates the 10 s blind spot (the headline contribution); the BW ceiling ensures the sender cannot overshoot the provisioned capacity of a beam it has never observed (the safety guarantee).

Relationship to draft-lai-ccwg-lsncc. BBR-SAT is motivated by the PPB problem statement [6]. Lai et al. define the discontinuous-path-change problem; we provide one concrete CCA mechanism that respects boundaries. If the CCWG or a future T4SAT working group standardizes boundary signaling, BBR-SAT demonstrates what a CCA-specific response to such signals looks like.

Signaling pathway scope. Predictive signaling (pathway 1) requires the transport endpoint within the operator's domain. For third-party endpoints (e.g., cloud servers), the reactive fallback (pathway 3) or INT (pathway 2) applies. The lead-time range study quantifies the performance cost of reactive vs predictive operation.

Controlled experiment limitations. picoquic's simulator does not model rain fade, scintillation, Doppler, or packet reordering. LEO channels exhibit significant micro-jitter [10]. Full-stack validation with LeoEM [10] or QUIC-NS + SNS-3 is deferred to a journal extension.

Cross-orbit handovers are rare. BBR-SAT's value concentrates in long-lived sessions spanning orbit classes — telemetry, IoT aggregation, asset tracking.

Relationship to Creo. Creo [5] handles intra-LEO with DRL+freeze. BBR-SAT handles cross-orbit with BDP context-switching. A combined mechanism is a natural future direction.

Complementarity with Careful Resume. CR accelerates connection start; BBR-SAT handles mid-connection path changes. Integration is future work.

Companion mechanisms. SCHC-based QUIC header compression and ephemeris-bound 0-RTT tokens are subjects of a separate publication.

# VII. Conclusion

We have proposed BBR-SAT, a minimal BBRv3 extension that maintains per-orbit-class (RTT, BW) tuples and switches between them on a handover signal, with an epistemological separation: RTT from ephemeris is trusted immediately; BW from ephemeris is a pacing-rate ceiling validated by ProbeBW. BBR-SAT fills a specific gap in the 2024–2026 literature: intra-LEO mechanisms freeze/pause but don't context-switch across orbit tiers; the IETF PPB abstraction [6] defines the boundary problem but specifies no CCA; Careful Resume is connection-start only. No prior work performs multi-tier BDP context-switching across orbit classes with an epistemologically grounded trust model. [PLACEHOLDER results summary.] Future work: integration with Creo, full-stack SNS-3 evaluation, companion publications on SCHC-QUIC and ephemeris-bound 0-RTT tokens.

# References

[1] J. Zhao, J. Pan, "StarQUIC," LEO-NET @ MobiCom 2024, DOI 10.1145/3697253.3697271.

[2] D. Zhao et al., "SatPipe," IEEE INFOCOM 2025, Xplore 11044600.

[3] Z. Lai et al., "LeoCC," ACM SIGCOMM 2025, pp. 129–146, DOI 10.1145/3718958.3750491.

[4] R. Valentine et al., "OrbCC," arXiv:2508.19067v2, Aug 2025. [Not peer-reviewed]

[5] Y. Yan, J. Li, J. Han, Q. Long, K. Xue, H. Chen, N. Qiao, "A Handover-Aware Congestion Control Algorithm Assisted by DRL in LEO Satellite Networks" (Creo), IEEE ICC 2025, Montreal, Xplore 11161022.

[6] Z. Lai, Z. Li, Q. Wu, H. Li, Q. Zhang, "Analysis for the Adverse Effects of LEO Mobility on Internet Congestion Control," draft-lai-ccwg-lsncc-01, Feb 2026 (CCWG, individual, Informational).

[7] N. Kuhn et al., "Convergence of Congestion Control from Retained State," draft-ietf-tsvwg-careful-resume-24, Oct 2025 (RFC Editor Queue).

[8] M. Hofstätter et al., "Careful Resume over Satellite Paths," IEEE ASMS/SPSC 2025, Xplore 10946055.

[9] N. Cardwell et al., "BBR Congestion Control," draft-ietf-ccwg-bbr-05, Mar 2026.

[10] T. Mazilu et al., "Evaluating Learning CC Schemes for LEO," arXiv:2510.25498, Oct 2025. [Not peer-reviewed]

[11] M. Duke, G. Fairhurst, "Specifying New Congestion Control Algorithms," RFC 9743 / BCP 133, Mar 2025 (obsoletes RFC 5033).

[12] K. Ohmori et al., "LEO Satellite-Friendly TCP CC," AINTEC 2025, DOI 10.1145/3763400.3763453.

[13] J. Garcia et al., "TCP CC Performance over Starlink," ANRW 2025, DOI 10.1145/3744200.3744760.

[14] H. Zhao, L. Chen, Y. C. Chou, H. Fang, J. Liu, "OrbitTransit," MobiSys 2026 Emerging Ideas, arXiv:2604.04368. (Simon Fraser University)

[15] T. Jones et al., "Enhancing Transport over Satellite," draft-jones-tsvwg-transport-for-satellite-02, Apr 2022 (expired).

[16] D. Yang, T. Tsou, "LEO Transport Problem Statement," draft-yang-tsvwg-leo-transport-problem-statement-00, Jun 2025 (expired).

[17] D. Yang et al., "LEO Transport Requirements," draft-yang-tsvwg-leo-transport-req-01, Jun 2025 (expired).

[18] D. Yue et al., "LEO Transport Use Cases," draft-yue-tsvwg-leo-transport-use-cases-00, Jul 2025 (expired).

[19] IRTF SPACERG, chartered Jan 2025, chairs J. Ott, J. Fraire.

[20] C. Huitema, "Picoquic," github.com/private-octopus/picoquic.

[21] Z. Lai et al., "Mind the Misleading Effects of LEO Mobility on CC," HotNets 2024, DOI 10.1145/3696348.3696867.

[22] D. Tiwari et al., "T3P," arXiv:2310.11835, Oct 2023. [Not peer-reviewed — NOT HotNets'24]

[23] N. Kuhn et al., "Getting up to Speed with Neqo and Careful Resume," ANRW 2025, DOI 10.1145/3744200.3744766.

[24] C. Huitema, M. Blanchet, "QUIC in Space," draft-huitema-quic-in-space-01, Jul 2025 (expired).

[25] M. Blanchet, "QUIC Profile for Deep Space," draft-many-tiptop-quic-profile-01, Aug 2025 (TIPTOP — excludes LEO/MEO/GEO ops).

[NOTE — All refs verified against IETF datatracker, IEEE Xplore, ACM DL, or arXiv as of May 2026.]

APPENDIX: Cumulative Change Log (v2→v5, Gemini R1–R3)

NOT FOR SUBMISSION

| # | Finding (Round) | Sev. | Disposition |
| --- | --- | --- | --- |
| 1 | BDP inversion: DRAIN trigger (R1) | CRIT | FIXED v3. DRAIN on BW_old > BW_new. |
| 2 | Creo prior art (R1) | HIGH | FIXED v4. Yan et al., ICC 2025. |
| 3 | Release 19 fatal flaw (R1) | HIGH | FIXED v3. Architecture-agnostic. |
| 4 | CR baseline = strawman (R1) | MED | FIXED v3. Reframed as IETF gap-proof. |
| 5 | Mazilu elevated (R1) | LOW | FIXED v3. |
| 6 | OrbitTransit misattribution (R2) | HIGH | FIXED v4. SFU, MobiSys EI Track. |
| 7 | Creo venue (R2) | MED | FIXED v4. ICC 2025 confirmed. |
| 8 | draft-lai-ccwg-lsncc missed (R2) | CRIT | FIXED v4. PPB cited, signal grounded. |
| 9 | Signaling pathway gap (R2) | CRIT | FIXED v4→v5. 3 pathways, (a) scoped to operator domain. |
| 10 | RFC 9743/BCP 133 missing (R2) | MED | FIXED v4. Obsoletes 5033 noted v5. |
| 11 | TSVWG cluster missing (R2) | MED | FIXED v4. |
| 12 | SPACERG missing (R2) | LOW | FIXED v4. |
| 13 | PPB framing = hijacking (R3) | HIGH | FIXED v5. 'Motivated by' not 'extending.' |
| 14 | Ephemeris→cloud signal gap (R3) | MED | FIXED v5. Pathway (a) scoped to operator. |
| 15 | BW not deterministic from ephemeris (R3) | CRIT | FIXED v5. RTT trusted, BW = ceiling + ProbeBW validation. |
| 16 | Missing fairness experiment (R3) | HIGH | FIXED v5. Exp 2 added (F1–F3, Jain's index). |
| 17 | RFC 9743 obsoletes 5033 (R3) | LOW | FIXED v5. Noted in Ref [11]. |